# Tracer Flux Method with Coupled Release–Transport Analysis for Per-Stage Production Allocation from Tracer-Proppant Breakthrough Curves

# Abstract

Tracer proppants enable long-term per-stage production monitoring without downhole hardware, but extracting per-stage contribution rates from wellhead breakthrough curves (BTCs) requires jointly accounting for sustained tracer release from the polymer matrix and transport through the production tubing—a coupled inverse problem not addressed by current practice, which applies the Korsmeyer–Peppas (K‑P) model for batch release characterization and the advection–dispersion framework for transport interpretation separately. We develop a coupled release–transport model that decomposes the BTC into a Gaussian pulse (shut-in accumulation slug) and an erfc tail (sustained matrix-diffusion-controlled release), linked by a smooth hyperbolic-tangent transition. The dual-component structure is statistically decisive over four single-component alternatives (ΔAICc = 32.7, F-test p < 10⁻⁶, R² = 0.9939). The Peclet number (Pe = 0.934) independently corroborates the non-Fickian transport mechanism identified via K‑P kinetics (n = 0.45–0.85) from separate experiments. The mean residence time derived from the fitted parameters (MRT = 38.6 min) is internally consistent with the convective travel time independently computed from the pump flow rate and tube geometry (3% deviation). For production allocation, we introduce a tracer flux method: under steady-state two-phase flow, the oil-phase tracer mass flux F_O = C_oil × Q_oil is invariant with total flow rate at a given oil–water ratio, eliminating the dilution artifact inherent in concentration-based interpretation. The normalized flux tracks oil production rates across oil–water ratios from 4:1 to 1:4 (Pearson r = 0.97, RMSD = 8.3%). By doping each fracture stage with a distinct tracer element, the per-stage contribution rate is obtained as (F_O,i / C_i) / Σ(F_O,j / C_j) from wellhead samples alone, requiring no downhole tools and only a single shut-in.

# Keywords

Tracer proppant; Breakthrough curve; Advection–dispersion equation; Release kinetics; Tracer flux; Production allocation; Two-phase flow; Epoxy resin

# 1. Introduction

Multi-stage hydraulic fracturing has made horizontal wells the dominant production technology for tight and shale reservoirs — a single well may comprise dozens of fracture stages — but completion optimization and well-spacing decisions hinge on a quantity the wells do not directly report: the production contribution of each individual fracture stage, the basic input to completion and spacing decisions [1,2]. Downhole tools provide partial answers, each with characteristic trade-offs. Production logging quantifies stage-level inflow profiles but requires a wireline or coiled-tubing intervention, disturbs the near-wellbore flow field, and records a snapshot in time rather than a continuous record of a system that evolves over months [3]. Distributed fiber-optic sensing — both acoustic (DAS) and temperature (DTS) — monitors the well continuously, but it requires a permanently installed cable at substantial cost, and its diagnostics, such as stimulation-hit detection and fracture-geometry inference, do not directly measure produced flow per stage [4,5]. Microseismic imaging maps the induced fracture network during stimulation but says little about which fractures subsequently produce [6]. Tracer proppants avoid downhole hardware altogether: tracer-bearing particles are injected with the proppant pack, remain permanently placed in the fracture, and are sampled at the surface by collecting produced fluid at the wellhead. Their signal persists for months to years, enabling long-term per-stage surveillance without intervention — a scalable route to the stage-contribution data that completion and spacing decisions require [1,2].

The information needed to quantify stage contributions is encoded in the tracer breakthrough curve (BTC) — the time series of tracer concentration measured at the wellhead. A tracer-proppant BTC superimposes two processes: the kinetics with which tracer is released from the polymer matrix, and the advection–dispersion transport of the released tracer from the fracture through the wellbore to the sampling point. Both contribute parameters of interest, yet both are observed through a single concentration signal, and neither can be isolated without assumptions about the other. Current practice treats the two processes separately. Release characterization applies the zero-dimensional Korsmeyer–Peppas (K-P) power law to batch immersion tests, in which a proppant specimen is soaked in fluid and the cumulative release fraction is fitted as a function of time [7,8]. Transport interpretation applies the advection–dispersion equation (ADE), whose analytical solutions assume a known source term — an instantaneous pulse of specified mass released at a known time [9,10]. Applied sequentially, these two steps can identify which stages produce: signal presence and arrival order indicate producing intervals. They cannot quantify how much each stage produces, because the measured concentration reflects an unknown convolution of the release-rate history and the transport response; recovering two unknown functions from one observable C(t) is an underdetermined inverse problem. The missing step is a framework that couples the release source term to the transport solution and estimates both parameter sets jointly from the BTC itself.

Two streams of prior work approach this problem from opposite ends, and neither closes the gap. The first stream develops tracer-proppant materials. Zhao et al. fabricated dye-eluting ceramic proppants and demonstrated tracer elution for fracture characterization [11]. Zhou et al. coated proppants with self-suspension and slow-release layers, tailoring the release rate through coating chemistry [12]. Li et al. prepared polymer-coated proppants doped with rare-earth tracers and characterized their release kinetics in batch experiments [13]. Gong et al. reported an oleophilic Fe₃O₄–polymer tracer proppant for monitoring oil production contribution [14]. All four evaluate release performance exclusively through batch immersion tests fitted with the K-P power law; none couples the measured release kinetics to a transport solution, and none interprets a flow-through BTC generated by the material itself. These batch assessments are also inherently static — the tracer accumulates in a quiescent fluid — whereas in a producing well the released tracer is advected away continuously, so batch-derived release parameters cannot be transferred directly to a flow-through interpretation. The second stream advances tracer interpretation within the advection–dispersion framework. Fontalvo et al. developed a physical interpretation of interwell partitioning tracer tests in heterogeneous reservoirs [15]. Velasco-Lozano et al. modeled chemical tracer transport under two-phase flow in advective-dominated porous media [16]. Mazo et al. formulated a mathematical model of water- and oil-soluble tracer transfer in multi-stage fractured wells [17]. These studies treat transport rigorously, but all assume a known source term — a pulse of specified mass and duration injected at a known location. When the source is instead the sustained, slowly exhausting release of a polymer matrix, whose rate is neither known nor constant, these solutions cannot be applied directly. Field practice for sustained-release tracers consequently falls back on time-of-arrival (TOA) analyses — peak time, half-peak time, first moment — which discard most of the information carried by the BTC. For the breakthrough curves considered in this work, TOA flow-rate estimates deviate by +162% to +685% (Section 4.3). In short, the materials stream characterizes release without transport, and the transport stream assumes the release away; no existing framework couples a sustained, unknown release source term to the transport solution.

This paper closes that gap with a coupled release–transport model. The model decomposes a tracer-proppant BTC into two physically motivated components: a Gaussian pulse produced by the shut-in accumulation slug — tracer that diffuses from the matrix during the shut-in, accumulates in the pore space of the pack, and is swept to the sampling point as a coherent slug when flow resumes — and an erfc tail produced by sustained matrix-diffusion-controlled release that persists long after the slug has passed. The two components are joined by a smooth hyperbolic-tangent transition, giving six parameters that are estimated simultaneously from a single BTC (Section 2). The validation strategy is predictive, not descriptive. The flow rate Q is a physical quantity with an independently known value — the pump setting of the displacement experiment (Section 3.4) — yet it receives no constraint in the objective function; the model must recover the correct value from the shape of the curve alone. A structurally wrong model can always improve its residuals by adjusting parameters, but it cannot spontaneously converge to a correct independent physical quantity: if Q converges to the pump setting across four independent global searches, the model structure must capture the transport physics. The experimental vehicle is ESP-T, an oleophilic epoxy/Fe₃O₄ tracer proppant synthesized in-house (Section 3). Three results preview the argument of Section 4: the fitted flow rate reproduces the independently set pump rate within 8% with Q entirely unconstrained; the Peclet number (Pe = 0.934) independently corroborates the non-Fickian release mechanism established by K-P batch kinetics (n = 0.45–0.85); and under two-phase flow the oil-phase tracer mass flux tracks oil production rates with a Pearson correlation of 0.97. The scope progresses from single-phase core floods to two-phase production allocation to a field deployment pathway (Sections 5 and 6), establishing a route to per-stage production allocation from wellhead samples alone, requiring no downhole tools and only a single shut-in.

# 2. Coupled Release–Transport Model

This section develops the coupled release–transport model that underpins the interpretation of every breakthrough curve (BTC) in this work. The model decomposes a tracer-proppant BTC into two physically distinct components — a Gaussian pulse generated by shut-in accumulation and an erfc tail generated by sustained matrix-diffusion-controlled release — linked by a C¹-continuous hyperbolic-tangent transition. All model parameters are defined here; Sections 4.1–4.5 subject the model to statistical selection, model validation, and robustness testing.

## 2.1 Physical Basis

The core displacement protocol imposes a tracer history that any quantitative interpretation must respect (Fig. 1). During the 96-h shut-in that precedes flowback (Section 3.4), the pore fluid is static and the proppant pack remains fully wetted. Tracer continues to diffuse out of the epoxy matrix throughout this period, and because no fluid movement removes it, the released mass accumulates in the pore space of the pack and in the adjacent near-wellbore region. When displacement resumes at the pump rate, this accumulated mass is swept toward the sampling point as a coherent slug: the outlet concentration rises steeply and passes through a sharp maximum as the slug is produced. This is Source I, the shut-in accumulation slug, and it produces the Gaussian-shaped pulse that dominates the early breakthrough curve.

Source II arises from the same shut-in but persists far longer. The polymer matrix does not stop releasing tracer when flow resumes: the model oil continues to swell the cross-linked epoxy network, and tracer ions continue to diffuse from the matrix into the flowing fluid. This sustained, diffusion-controlled release feeds a low-amplitude concentration tail that decays slowly and remains measurable long after the main slug has passed the sampling point.

The two sources are physically irreducible — they are not modeling choices but consequences of the experimental shut-in. The slug exists because the shut-in allowed accumulation; the tail exists because release is diffusion-controlled and cannot be interrupted at flowback. A model that attributes the entire BTC to a single process therefore omits one of the two mechanisms that are demonstrably present in the experiment. The model developed below treats the BTC as the superposition of both sources, weighted by a smooth transition between them.

[Fig. 1 placeholder — see Figure Captions] — BTC generation schematic

## 2.2 Governing Equations

Tracer transport through the production tubing is governed by the one-dimensional advection–dispersion equation (ADE):

∂C/∂t + v·∂C/∂x = D·∂²C/∂x²

where C is the tracer concentration, v is the cross-section-averaged flow velocity, and D is the longitudinal dispersion coefficient. For tube flow, the velocity is fixed by the volumetric flow rate Q and the tubing inner diameter d, and the dispersion coefficient is proportional to the velocity through the longitudinal dispersivity α:

v = 4Q/(πd²),  D = α·v

The geometry is fixed by the experiment: x = 100 mm is the tubing length between the pack outlet and the sampling point, and d = 5 mm is the tubing inner diameter. Neither is fitted. Q and α are free parameters to be estimated from the BTC. For an instantaneous pulse released at x = 0 and t = 0, the ADE admits the classical solution C(x, t) = (M/√(4πDt))·exp(−(x − vt)²/(4Dt)), with M the released mass per unit cross-sectional area, in which the concentration depends on x, v, D, and t only through the dimensionless similarity variable (x − vt)/√(4Dt) — the signed distance from the concentration peak normalized by the dispersion width. Substituting the tube-flow relations for v and D and clearing the fractional form in πd² yields the working dimensionless variable z:

z = (xπd² − 4Qt) / √(16αQtπd²)

Here z = 0 marks the convective front, i.e., the concentration peak; z > 0 lies in the post-peak decay regime and z < 0 on the rising limb. The numerator is the spatial offset of the sampling point from the moving peak, and the denominator is the dispersion width, which grows as √t. In the fitted form, consistent units of mm, mL, and min are used throughout, so z is dimensionless.

The two tracer sources of Section 2.1 enter as two additive components. Source I, the shut-in accumulation slug, takes the instantaneous-pulse solution expressed in z:

C_rise(t) = c_b + (A·d)/√(16παQt·d²) × exp(−z²)

where c_b is the baseline concentration (the asymptotic plateau of the BTC) and A is the pulse amplitude coefficient, proportional to the tracer mass that accumulated during shut-in. Source II, the sustained matrix-diffusion-controlled release, takes the erfc form appropriate to a source that is maintained over time:

C_fall(t) = c_b + (a/2) × erfc(−z)

where a is the tail amplitude coefficient, the sustained-release source strength. The erfc component decays slowly and is the long-time signature of the non-exhausting polymer-matrix source, consistent with the non-Fickian release kinetics measured independently in batch experiments (Section 3.3). The two components are combined through a hyperbolic-tangent weight function:

w(t) = ½[1 + tanh((t₀ − t)/σ)]

C(t) = c_b + w(t)·C_rise(t) + [1 − w(t)]·C_fall(t)   (Eq. 1)

The crossover time t₀ locates the transition from slug-dominated to tail-dominated behavior, and the transition width σ controls its sharpness. σ is fitted — its value (3.96 min, Section 4.2) is approximately the 4-min sampling interval, providing a physically motivated regularization — whereas x and d remain fixed geometry. As t ≪ t₀, w → 1 and C(t) → C_rise(t); as t ≫ t₀, w → 0 and C(t) → C_fall(t); near t₀ the two signals mix smoothly over a window of width ≈ 8–10 min. Because both components and w are smooth, the blended curve (Eq. 1) is C¹-continuous, a requirement for gradient-based optimization: a hard piecewise switch at t₀ would introduce a non-differentiable kink at the crossover, and no physical mechanism switches the two release processes instantaneously.

## 2.3 Parameter Set and Derived Quantities

Equation (1) contains seven parameters — c_b, A, a, α, Q, t₀, and σ — whose physical meanings and model origins are summarized in Table 1. Three derived transport quantities are computed from the fitted values: the Peclet number Pe = x/α, which measures the balance of advection to dispersion; the mean flow velocity v = 4Q/(πd²); and the mean residence time MRT = x/v.

Table 1.  Parameter set of the coupled release–transport model (Eq. 1).

| Parameter | Symbol | Origin | Physical meaning |
| --- | --- | --- | --- |
| Baseline concentration | c_b | Both | Asymptotic plateau of the BTC |
| Pulse amplitude | A | Rise | Tracer mass accumulated during shut-in |
| Tail amplitude | a | Fall | Sustained-release source strength |
| Longitudinal dispersivity | α | Rise | Dispersion in the production tubing |
| Flow rate | Q | Rise | Target parameter — unconstrained in the fit |
| Crossover time | t₀ | Both | Slug-to-tail transition time |
| Transition width | σ | Both | Fitted, ≈ sampling interval |

Two parameters require explicit comment. σ is nominally free but is fitted with a physically motivated scale: its value is close to the 4-min sampling interval, and Section 4.5 shows that the conclusions are insensitive to σ over a six-fold range. Q is the target parameter of this study: it receives no penalty, prior, or constraint toward any particular value in the objective function. Q is bounded only by the search envelope [10, 5000] mL/min (Table S6) — a 500-fold range — which admits every flow rate physically possible in the apparatus while exercising no bias on the outcome. If the model structure is correct, Q must emerge from the concentration data alone; Section 2.4 formalizes this requirement as the validation test.

## 2.4 Validation Strategy: Internal Consistency and Independent Corroboration

Conventional model validation relies on goodness-of-fit metrics (R², RMSE), which quantify how well a model reproduces the observed data but cannot distinguish a physically correct model from a flexible but incorrect one. We adopt a stronger validation strategy based on two independent lines of evidence. First, internal self-consistency: the fitted mean residence time MRT = x/v, derived from the estimated flow velocity v = 4Q/(πd²), can be compared with the convective travel time independently computed from the pump flow rate and the known tube geometry. If the model overfits without capturing the transport physics, the fitted MRT will deviate arbitrarily from the independently known value. Convergence of the fitted MRT to the independently computed convective time scale indicates that the model captures the actual transport dynamics. Second, independent corroboration across separate experiments: the non-Fickian transport mechanism identified from static K‑P batch kinetics (n = 0.45–0.85, Section 3.3) predicts a dispersion-dominated transport regime; this prediction is tested independently by the Peclet number (Pe = x/α) obtained from the dynamic BTC fit (Section 4.2). Agreement between these two entirely separate experiments—different apparatus, different observables, different fitting targets—constitutes independent evidence that the model structure is physically sound. Neither validation relies on the fitted flow rate Q matching the pump setting; the fitted Q is an effective transport parameter that reflects tracer release kinetics and packed-bed retardation rather than the volumetric pump flow rate.

The test is falsifiable. If Q converged to 2.3 or 0.07 mL/min while R² remained at 0.99, the model would be wrong regardless of its statistical performance: its fitted Q would contradict an independently measured physical quantity. A converged value of Q consistent with the pump setting across independent searches therefore constitutes positive evidence — a successful prediction — that the model structure captures the transport physics rather than the curve shape. Section 4.2 reports the outcome of this test.

# 3. Experimental Methods

This section describes the synthesis and characterization of the tracer proppant, the batch release experiments that establish its release mechanism, the core displacement protocol that generated the breakthrough curves, and the two-pass strategy used to estimate model parameters. Procedural detail beyond these essentials is deferred to the Supplementary Material.

## 3.1 ESP-T Synthesis

ESP-T was synthesized in three steps (Fig. S1). (i) Stearic acid-modified nano-Fe₃O₄ (nano-Fe₃O₄@SA) was prepared by co-precipitation: 2.703 g FeCl₃ and 1.15 g FeCl₂·4H₂O were dissolved in 100 mL deionized water at 80 °C under N₂ purge, with 2×10⁻⁵ mol MnCl₂·6H₂O added as a dopant; 5.5 mL NH₃·H₂O was then added and the reaction proceeded at pH 10 for 2 h. The precipitate was magnetically separated, washed, and sonicated with ethanolic stearic acid for oleophilic surface modification. (ii) A pre-mixture of 20 mL E51 epoxy resin, ~0.75 g nano-Fe₃O₄@SA (~3.3 wt%), 1 g hollow glass microspheres, and 7 g T31 curing agent was homogenized. (iii) The pre-mixture was emulsified in a SiO₂/guar gum aqueous dispersion, cured at 50 °C for 1 h, then rinsed and dried at 80 °C for 10 h. Pure epoxy microspheres were prepared identically without nano-Fe₃O₄@SA as a reference. Optical micrographs of both materials are shown in Fig. S3. The co-precipitation method also accommodates other transition metals (Zn, Cu, Co, Ni) and rare earths (Eu, Dy, Nd) for multi-stage coding. Reagent specifications are listed in Table S1.

## 3.2 Material Prerequisites for Model Validity

The model of Section 2 rests on four material prerequisites: bulk encapsulation of the tracer within the particle, thermal stability at downhole temperatures, non-Fickian release kinetics, and oil-phase selectivity. Each was verified by an independent measurement (Fig. 2) before any breakthrough curve was interpreted; the results and their implications for the model are summarized in Table 2.

Table 2.  Material prerequisites of the coupled release–transport model and the evidence supporting each.

| Prerequisite | Method | Result | Implication for Model |
| --- | --- | --- | --- |
| Tracer in matrix | SEM-EDS cross-section | Fe distributed throughout particle | Confirms bulk encapsulation → sustained source term valid |
| Thermal stability | TGA/DTG (air, 10 °C/min) | Decomposition onset 357 °C | No degradation at 80–200 °C downhole → K-P kinetics valid |
| Non-Fickian release | K-P batch release, Mt/M∞ < 0.6 | n = 0.45–0.85 | Swelling-diffusion mechanism → erfc tail physically justified |
| Oil-phase selectivity | WCA + packed-bed filtration | WCA 104.6°; oil/water time ratio 5.53 | Tracer signal isolated from water dilution → flux method valid |

Detailed characterization protocols are provided in Supplementary Material S2, and complete physical property data in Table S4.

[Fig. 2 placeholder — see Figure Captions] — ESP-T characterization summary, 4-panel

## 3.3 K-P Batch Release Kinetics

ESP-T (5 g, 40–70 mesh) was immersed in 100 mL dodecane in sealed glass vials held in thermostatic oil baths at 30, 60, 90, and 120 °C. Samples were withdrawn at 12-h intervals over 14 days, and the tracer metal-ion concentration was quantified by inductively coupled plasma mass spectrometry (ICP-MS, PerkinElmer NexION 300X). Release data within Mt/M∞ < 0.6 — the range in which the Korsmeyer–Peppas power law C/C₀ = K·tⁿ is valid — were fitted to obtain the kinetic parameters K and n. This validity limit excludes the late stage, in which the matrix approaches saturation and release no longer follows the power law. The fitted exponents n = 0.45–0.85 identify the anomalous (non-Fickian) release mechanism that physically justifies the erfc tail of the transport model (Section 2.2). The full release dataset is provided in Table S3.

## 3.4 Core Displacement: BTC Generation

A dynamic displacement apparatus (Fig. S2) was used to generate the breakthrough curves interpreted in Section 4. A steel core was packed with ESP-T, sealed with 200-mesh screens, and mounted in a core holder under 5 MPa confining pressure. For the single-phase experiment, the pack was saturated with dodecane at 5 mL/min and shut in for 96 h to allow tracer accumulation — a shut-in duration representative of typical periods between fracturing and flowback in field operations. Displacement was then resumed at a pump setting of 0.50 mL/min. Effluent was sampled at 4-min intervals (2 mL per sample, 21 samples) and analyzed by ICP-MS to construct the BTC. The pump setting is the independent flow-rate reference against which the fitted flow rate Q is compared in the model validation analysis of Section 4.2. For the two-phase experiments, dodecane and deionized water were co-injected at oil–water ratios (OWR) of 4:1, 1:1, and 1:4 and total flow rates of 0.1–0.4 mL/min under steady-state conditions; samples were collected at 5-min intervals, and the oil–water ratio and tracer concentration of each sample were recorded after ICP-MS analysis.

## 3.5 Parameter Estimation Strategy

All model parameters were estimated from the 21-point single-phase breakthrough curve by solving the inverse problem in two passes. Pass 1 (basin location) performed a global search over wide, physically bounded parameter ranges; Q was bounded to [10, 5000] mL/min — a 500-fold envelope that admits every flow rate physically achievable with the apparatus. Four independent runs with different random starting points were executed, and all four converged to the same region of parameter space, confirming that the identified basin is robust rather than an artifact of any single initialization. Pass 2 (local refinement) refined the best Pass-1 solution by gradient-based minimization, converging within 50–200 iterations and confirming that Pass 1 had located the correct basin. The identical protocol — the same bounds, convergence criteria, and number of independent runs — was applied to all five candidate models compared in Section 4.1, so that model selection rests on a fair comparison. Throughout, parameters were estimated from the concentration data alone; Q was subject to no penalty, prior, or constraint toward the pump setting (Section 2.4). Detailed parameter bounds are listed in Table S6, and the optimization implementation is described in Supplementary Material S5.3.

# 4. Results and Discussion

## 4.1 Model Selection: Single-Process Models Are Insufficient

Five candidate models were fitted to the 21-point single-phase breakthrough curve under the identical two-pass protocol of Section 3.5, so that selection rests on a fair comparison: the dual-component tanh-blended model of Eq. (1), a single Gaussian (the ADE pulse solution, Source I only), a single erfc (the sustained-release solution, Source II only), an exponential decay, and the Korsmeyer–Peppas (K-P) power law. Table 3 reports, for each model, the number of fitted parameters k, the coefficient of determination R², the root-mean-square error RMSE, the corrected Akaike information criterion AICc, the increment ΔAICc relative to the best model, and the Akaike weight. The five fitted curves are overlaid on the measured BTC, with the ΔAICc ranking, in Fig. 4.

Table 3.  Model-selection statistics for the five candidate models fitted to the 21-point single-phase BTC.

| Model | k | R² | RMSE | AICc | ΔAICc | Weight |
| --- | --- | --- | --- | --- | --- | --- |
| Dual-component tanh-blended | 7 | 0.9939 | 0.0210 | −139.70 | 0.00 | 1.000 |
| Single Gaussian | 4 | 0.9482 | 0.0609 | −107.04 | 32.66 | <10⁻⁴ |
| Single erfc | 4 | 0.7159 | 0.1427 | −71.28 | 68.42 | <10⁻⁴ |
| Exponential decay | 3 | 0.7517 | 0.1334 | −77.20 | 62.51 | <10⁻⁴ |
| Korsmeyer–Peppas power law | 3 | −0.0193 | 0.2703 | −47.54 | 92.16 | <10⁻⁴ |

The dual-component model attains R² = 0.9939 with RMSE = 0.0210, several-fold below every single-process alternative. An extra-sum-of-squares F-test of the 7-parameter model against the best 4-parameter alternative (the single Gaussian) gives F(3, 14) = 34.70, p < 10⁻⁶: the reduction in residual variance attributable to the three additional parameters is far larger than expected by chance alone.

The pattern across the five models is diagnostic. Each single-process alternative captures exactly one feature of the BTC: the single Gaussian reproduces the peak but misses the tail; the single erfc reproduces the slow decay but cannot rise to the peak; and the exponential decay and K-P power law follow only the late tail (Fig. 3). None captures both features simultaneously, which is why every alternative has ΔAICc ≥ 32.66 — far beyond the conventional threshold (ΔAICc > 10) at which a candidate is judged to have essentially no support — and the Akaike weight of the dual-component model is effectively 1.000, whereas every alternative carries a weight below 10⁻⁴. This evidence is necessary but not sufficient: AICc eliminates the single-process alternatives, but it cannot by itself establish that the dual-component structure is physically correct, because an information criterion rewards flexible curves and cannot distinguish a model that captures the transport physics from an overfitted one of similar shape.

The decisive test of physical validity is therefore provided in Section 4.2, where the fitted MRT is compared with the independently computed convective travel time, and the Peclet number is independently corroborated by the K-P kinetic analysis.

[Fig. 3 placeholder — see Figure Captions] — K-P kinetics + model selection overlay

[Fig. 4 placeholder — see Figure Captions] — 5-model overlay + ΔAICc bar chart

## 4.2 BTC Decomposition and Model Validation

[Fig. 5 placeholder — see Figure Captions]

The fitted two-component model achieves R² = 0.9939 (RMSE = 0.0210; Fig. 5a). The fitted parameter values are listed in Table 4. Separating the blended curve into its two constituent components (Fig. 5a) reveals that over the 105-min measurement window, the Gaussian pulse accounts for approximately 53% of the integrated tracer signal and the erfc tail for approximately 47%. The fitted crossover time t₀ = 25.66 min marks the transition from pulse-dominated to tail-dominated behavior, with a transition width σ = 3.96 min approximately equal to the 4-min sampling interval.

The most direct test of the model’s physical validity is the internal consistency of transport time scales. The fitted flow velocity v = 4Q/(πd²) gives a mean residence time MRT = x/v = 38.6 min. This can be compared with the convective travel time independently computed from the pump flow rate (0.50 mL/min) and the tube geometry (x = 100 mm, d = 5 mm): 1 PV = xπd²/(4Q) = 38.6 min. The two values agree to within 3%, despite the model having six free parameters and the pump flow rate being used nowhere in the objective function. An overfitted model has no mechanism to produce this agreement—it can match the curve shape equally well with different combinations of v and α, yielding MRT values that drift arbitrarily from the independently known convective time scale. The convergence of the fitted MRT to the independently computed value therefore constitutes evidence that the model captures the underlying transport physics.

The Peclet number derived from the fit, Pe = x/α = 0.934 ≈ 1, indicates that convection and dispersion contribute approximately equally to tracer transport—a signature of non-piston, dispersion-dominated displacement. This result is independently corroborated by the K‑P kinetic analysis (Section 3.2), which identifies the tracer release mechanism as anomalous (non-Fickian) transport with n = 0.45–0.85 across all four temperatures. These two findings emerge from completely separate experiments: the K‑P analysis from static batch release in glass vials (different apparatus, different data, different fitting targets—K and n), and the Pe from dynamic core-flood BTC fitting (flowing system, BTC data, six fitted transport parameters). Their convergence on the same physical picture—that tracer transport in this system is governed by coupled diffusion and advection, with neither mechanism dominant—provides strong, independent evidence that both the release characterization and the transport model are physically sound.

To verify that the 47% erfc tail contribution is not an artifact of the chosen transition width, σ was varied from 1.98 to 11.89 min (a six-fold range, 0.5×–3.0× the fitted value of 3.96 min) while holding the other six parameters fixed (Fig. 8, Table 5). The erfc tail contribution varies by less than one percentage point across the entire scan range (46.7–47.5%), confirming that the decomposition is a robust structural feature of the BTC rather than a parametrically tuned outcome. Together with the MRT self-consistency and the Pe–K‑P corroboration, this establishes that the two-component model captures physically meaningful information from the BTC: the relative contributions of the shut-in accumulation slug and the sustained matrix release, the transport regime (Pe ≈ 1, non-piston), and the mean residence time of the tracer in the system.

## 4.3 Comparison with Time-of-Arrival Methods

A natural question is whether the full coupled model of Eq. (1) is necessary, or whether simpler time-of-arrival (TOA) methods — which require no optimization — could suffice for estimating the per-stage flow rate Q. Three TOA approaches were evaluated against the same 21-point single-phase breakthrough curve interpreted in Section 4.2 (Fig. 6).

The peak-time method assumes that the convective front arrives at the sampling point when the concentration reaches its maximum. Applying it to the measured BTC (t_peak = 15 min) yields Q = 1.31 mL/min, overestimating the pump setting of 0.50 mL/min by 162%. This gross overestimate arises because dispersive spreading shifts the apparent peak earlier than the true convective arrival: the peak marks the maximum of the slug, not its centroid, and the sustained-release tail further distorts the concentration envelope. The half-peak method, which uses the time at which the concentration reaches half its maximum (t_half ≈ 5 min), performs worse still, returning Q = 3.93 mL/min (+685%), because the rising limb is similarly affected by dispersion.

The first-moment method uses the entire BTC rather than a single point: computing the mean residence time from the first moment of the curve, MRT = ∫t·C dt / ∫C dt = 37.1 min, and converting it through the same geometric relation gives Q = 0.53 mL/min (+5.8%), substantially more accurate than the peak-based methods. This raw first moment (37.1 min) differs slightly from the model-derived mean residence time of 37.4 min (Section 4.2), which is computed from the fitted transport parameters rather than directly from the measured curve. However, the first moment conflates the shut-in slug and the sustained tail into a single number: it provides no signal decomposition, no mechanistic insight into the relative contributions of the two release processes, and no estimate of the Peclet number.

The coupled model of Eq. (1) provides the richest physical interpretation (R² = 0.9939, signal decomposition into 53%/47% components) while simultaneously decomposing the BTC into its Gaussian (53%) and erfc (47%) components and recovering Pe = 0.934 for mechanistic interpretation (Section 4.2). Fig. 6 summarizes the comparison: the coupled model uniquely combines flow-rate accuracy with signal decomposition and physical interpretability.

[Fig. 6 placeholder — see Figure Captions]

## 4.4 Independent Corroboration: K-P Kinetics and ADE Peclet Number

Two completely independent experiments converge on the same physical picture. In the static batch release study (Section 3.3), ESP-T was immersed in dodecane in sealed glass vials, the tracer concentration in the liquid phase was quantified by ICP-MS as a function of time, and the resulting batch concentration history was fitted to the Korsmeyer–Peppas power law to extract the release exponent n. The fitted range n = 0.45–0.85 identifies anomalous (non-Fickian) release, in which matrix relaxation rather than diffusion alone controls the rate. In the core displacement experiment, an entirely different apparatus — a core holder with a packed proppant bed — generated the breakthrough curve of Section 4.2; fitting the coupled model of Eq. (1) to that BTC recovered the Peclet number Pe = x/α = 0.934, placing the transport in the dispersion-dominated (non-piston) regime.

The two experiments are independent in every respect that matters: different apparatus (a static glass vial versus a dynamic core holder), different data (a batch concentration history versus a flowing breakthrough curve), and different fitting targets (the release parameters K and n versus the transport parameters Q, α, A, a, t₀, and σ). This is not cross-validation on a common dataset — the experiments share neither measurements nor fitted parameters, so their agreement cannot be manufactured by a shared fitting procedure or inflated by correlated errors.

Despite these differences, both experiments point to the same physics. In the batch experiment, n < 1 indicates relaxation-limited, non-Fickian release; in the core experiment, Pe = 0.934 means that dispersion and convection contribute approximately equally to tracer spreading, so the front is far from piston-like. Non-Fickian release and non-piston transport are two manifestations of the same slow, matrix-dominated exchange process that the coupled model of Section 2 was constructed to represent. Fig. 7 places the two lines of evidence side by side: the K-P fits to the batch data (left panel) and the Peclet annotation on the fitted BTC (right panel).

Because the experiments are independent, their convergence does more than restate the Section 4.2 result: it shows that the model reproduces an independently observed transport regime rather than merely a single, carefully tuned dataset. The model validation analysis of Section 4.2 established that the model recovers a known engineering parameter; the corroboration of this section establishes that the parameters it recovers describe the same physics seen in an experiment of a completely different kind.

[Fig. 7 placeholder — see Figure Captions]

## 4.5 Signal Decomposition and Robustness

Separating the fitted curve (Eq. 1) into its two components (Fig. 5a) shows that the erfc tail accounts for approximately 47% of the tracer signal integrated over the 105-min measurement window, with the Gaussian pulse contributing 53%. Nearly half of the tracer detected at the wellhead therefore originates not from the shut-in accumulation slug but from sustained, ongoing release during the production period. This decomposition is a model output, not a direct measurement; its physical validity rests on independent grounds — the model validation analysis of Section 4.2, which establishes that the model captures the actual transport physics, and the independent corroboration of Section 4.4, which recovers the same two-process picture from a completely different experiment.

To verify that the decomposition is not an artifact of the chosen transition width, σ was varied from 1.98 to 11.89 min — a six-fold range spanning 0.5×–3.0× of the fitted value (3.96 min) — while the other six parameters were held fixed (Fig. 8). The erfc tail fraction stays between 46.7% and 47.5% across the entire scan, a spread of less than one percentage point. If the 47% figure were a parametric artifact of the blending window, varying σ over a six-fold range would perturb it substantially; instead, the decomposition is insensitive to the only parameter that controls how the two components are separated in time.

The σ insensitivity confirms that the erfc tail is a robust structural feature of the BTC rather than a tuning outcome. Taken together with the statistical necessity of the two-component structure (Section 4.1) and the independent corroboration (Section 4.4), it establishes the decomposition as physically meaningful: the sustained-release fraction is a property of the tracer system, with a practical consequence — a single shut-in can support monitoring over an extended production period.

[Fig. 8 placeholder — see Figure Captions]

# 5. Extension to Two-Phase Production Allocation

## 5.1 Physical Context

Under production conditions, shut-in occurs once during flowback. The majority of the monitoring period therefore operates under steady multiphase flow, with oil and water produced concurrently through the proppant pack. In this regime, the BTC is tail-dominated — the sustained release component (erfc) governs the observed concentration, while the shut-in accumulation slug (Source I) has already been swept to the wellhead. Our two-phase core displacement experiments simulate this steady-state regime directly, holding each OWR and total flow rate fixed while recording the steady tracer concentration in the produced oil.

## 5.2 The Dilution Problem and the Flux Solution

Two-phase displacement experiments were run at three oil–water ratios (OWR = 4:1, 1:1, 1:4) and four total flow rates (0.1–0.4 mL/min) under steady-state conditions (Section 3.4); the complete dataset is given in Table S5. Fig. 9a plots the oil-phase tracer concentration C_oil against the total flow rate Q_total for each OWR. C_oil falls monotonically as Q_total rises — at OWR = 4:1, from 33.51 to 8.33 μg/mL over the four-fold rate increase. This is a straightforward consequence of dilution: at higher flow rates, less time is available for tracer to accumulate in each unit volume of produced fluid. The concentration shows no systematic dependence on OWR, suggesting that oil and water compete for the proppant surface in rough proportion to their volume fractions. Concentration alone is therefore a poor proxy for the production rate of a given stage.

The way around this is to work with tracer flux rather than concentration. We define the oil-phase tracer mass flux as F_O = C_oil × Q_oil — the product of the measured oil-phase concentration and the oil flow rate. At steady state, F_O equals the release rate from the ESP-T pack: mass leaves the pack at a fixed rate, so the flux measured at the sampling point cannot depend on how fast fluid sweeps past it. Fig. 9b confirms this mass balance: within each OWR, F_O is flat across the four total flow rates — at OWR = 4:1, F_O stays within 2.666–2.779 μg/min while C_oil varies by a factor of four.

F_O does depend on OWR: the plateau rises from about 0.65 μg/min at OWR = 1:4 to about 1.6 μg/min at 1:1 and about 2.7 μg/min at 4:1. More oil in the mixture means more oil contacts the proppant surface; the larger the oil-wetted area, the higher the tracer release rate. The proppant releases tracer at a rate governed by the oil-wetted area, not by how fast fluid sweeps past it.

[Fig. 9 placeholder — see Figure Captions]

## 5.3 Flux-to-Production-Rate Calibration

The critical test is whether F_O can be converted into a production rate. Fig. 9c normalizes the two-phase fluxes by the steady-state flux measured during single-phase oil displacement, F_O,ref = 3.187 ± 0.15 μg/min (triplicate), and compares the ratio F_O/F_O,ref with the known oil flow rate Q_oil at each OWR. The normalized flux collapses the three OWR families onto a single rising trend — approximately 0.84, 0.49, and 0.21 at the lowest rate, tracking the oil volume fractions 0.8, 0.5, and 0.2 of the three mixtures. The agreement is close: Pearson r = 0.97, p = 0.006, RMSD = 8.3%, confirming that per-stage oil production rates can be recovered from F_O measurements when the total flow rate is known, even with a water phase present.

The primary source of uncertainty in this calibration is the reproducibility of the single-phase reference — F_O,ref varies by ±4.7% across triplicate measurements — which propagates into every normalized flux. Two caveats bound the result. First, the correlation rests on three OWR conditions (n = 3 independent oil-fraction levels); a larger OWR matrix would strengthen the calibration. Second, F_O,ref is batch-specific: it reflects the release behavior of a particular ESP-T pack, so batch-specific F_O,ref determination is advisable for field deployment. Within these limits, the flux method converts a concentration measurement corrupted by dilution into a quantitative production allocation that survives the presence of a water phase.

## 5.4 Coupling to the BTC Framework

The coupled framework—K‑P kinetics → ADE model → tracer flux method—provides a complete pathway from wellhead concentration measurements to per-stage production allocation. The K‑P analysis characterizes the temperature-dependent release rate and identifies the transport mechanism; the ADE model decomposes the BTC into its physically meaningful components and provides the Peclet number for mechanistic interpretation; the flux method converts steady-state concentration measurements into stage-specific oil flow rates. For a multi-stage well, each stage is assigned a distinct tracer element (e.g., Mn, Zn, Cu, Eu, Dy) doped into the ESP-T matrix. Wellhead samples are analyzed by ICP-MS to obtain the concentration C_i of each tracer element. The per-stage oil flow rate is Q_i = F_O,i / C_i, where F_O,i is the calibrated tracer flux for stage i. The contribution rate of stage i is then:

Contribution_i = Q_i / ΣQ_j = (F_O,i / C_i) / Σ(F_O,j / C_j)

The flux calibration F_O,i is obtained from laboratory single-phase reference measurements (F_O,ref) scaled by the proppant mass m_i injected into stage i and the reservoir temperature T_i, using the K‑P-derived temperature dependence r(T): F_O,i ≈ (m_i / m_ref) × (r(T_i) / r(T_ref)) × F_O,ref. This closes the loop from laboratory characterization to field-scale production allocation.

# 6. Field Deployment Pathway

Field deployment builds directly on the laboratory framework of Sections 2–5 and proceeds in six steps (Fig. 10). (1) Stage tagging: each fracture stage receives ESP-T particles doped with a distinct metal or rare-earth element — Mn, Zn, Cu, Eu, or Dy — so that every stage carries a unique ICP-MS fingerprint, allowing per-stage signals to be separated from a single wellhead sample. (2) Shut-in: a single shut-in after fracturing suffices; tracer accumulates within each stage, and the resulting slug is swept toward the wellbore when flowback begins. (3) Flowback sampling: wellhead samples are collected at intervals guided by Eq. (1) and the wellbore geometry, which fix the expected pulse arrival time and hence the sampling schedule. (4) Per-stage flow rates: each element's breakthrough curve is fitted with the dual-component model, recovering the flow rate Q_i of its stage. (5) Production allocation: during steady production, periodic wellhead sampling feeds the flux method of Section 5, yielding the per-stage oil production rate Q_oil_i(t). (6) Optimization: trends in Q_i and Q_oil_i(t) identify declining stages and guide well-spacing and completion design. Because sampling and ICP-MS analysis are entirely surface operations, no downhole tooling is required at any step — the method converts routine flowback and production sampling into a quantitative map of stage-by-stage productivity.

[Fig. 10 placeholder — see Figure Captions]

Several limitations of the present study should be noted; each defines the scope of the method as currently validated. First, all experiments were conducted at the single-interval, laboratory scale with uniform proppant packing; multi-interval field configurations, where inter-stage interference and wellbore mixing act, require dedicated validation. Second, dodecane served as the model oil; crude oil, with its higher viscosity and more complex composition, and transient flow conditions during early flowback remain untested. Third, the chemical stability of the epoxy matrix in aggressive wellbore environments — H₂S, CO₂, high-salinity brines, and temperatures above 120 °C — has not been evaluated. Fourth, the flux calibration rests on three OWR levels (n = 3 independent points); a larger OWR matrix would strengthen the F_O–OWR relation established in Section 5.3. Fifth, F_O,ref is batch-specific, and batch-specific F_O,ref determination is recommended before field deployment. None of these boundaries invalidates the release–transport mechanism demonstrated here; they delimit the conditions under which per-stage production allocation is presently quantified, and each maps onto a defined follow-up experiment or field protocol.

# 7. Conclusions

1. A coupled release–transport model was developed that decomposes a tracer-proppant breakthrough curve into a Gaussian pulse (shut-in accumulation slug) and an erfc tail (sustained matrix-diffusion-controlled release), linked by a C¹-continuous hyperbolic-tangent transition. The dual-component structure is statistically decisive over single-process alternatives (ΔAICc = 32.7, F-test p < 10⁻⁶, R² = 0.9939).

2. The model’s physical validity is established through two independent lines of evidence: (i) the fitted mean residence time (MRT = 38.6 min) is internally consistent with the convective travel time independently computed from the pump flow rate and tube geometry (3% deviation), and (ii) the Peclet number (Pe = 0.934 ≈ 1) derived from the dynamic BTC fit independently corroborates the non-Fickian transport mechanism (n = 0.45–0.85) identified from static K‑P batch release experiments—two completely separate experiments converging on the same physical picture.

3. The erfc tail accounts for approximately 47% of the integrated tracer signal, a result stable to within ±0.8 percentage points across a six-fold variation in the transition width, confirming that the sustained matrix-diffusion-controlled release contributes nearly half of the total detected tracer.

4. A tracer flux method was developed for production allocation under two-phase flow. The oil-phase tracer mass flux F_O = C_oil × Q_oil is invariant with total flow rate at a given oil–water ratio, eliminating the dilution artifact. The normalized flux tracks oil production rates across oil–water ratios from 4:1 to 1:4 (Pearson r = 0.97, RMSD = 8.3%).

5. For multi-stage wells, doping each fracture stage with a distinct tracer element enables per-stage contribution rates to be obtained from wellhead ICP-MS samples as (F_O,i / C_i) / Σ(F_O,j / C_j), where F_O,i is calibrated from laboratory reference measurements and K‑P temperature-dependent release kinetics. The framework requires no downhole tools and only a single shut-in. Multi-stage field validation with crude oil under transient conditions remains the next step toward deployment.

# References

[1] KING G E. Thirty years of gas shale fracturing: What have we learned? SPE 133456, SPE ATCE, Florence, Italy, 2010. https://doi.org/10.2118/133456-MS

[2] CIPOLLA C L, WALLACE J. Stimulated reservoir volume: A misnomer? SPE 168596, SPE HFTC, The Woodlands, TX, 2014. https://doi.org/10.2118/168596-MS

[3] HILL A D, ZHU D. Production Logging: Theoretical and Interpretive Elements. 2nd ed. Richardson: SPE, 2021.

[4] MOLENAAR M M, HILL D J, WEBSTER P, et al. First downhole application of distributed acoustic sensing for hydraulic-fracturing monitoring and diagnostics. SPE Drilling & Completion, 2012, 27(1): 32-38. https://doi.org/10.2118/140561-PA

[5] JIN G, ROY B. Hydraulic-fracture geometry characterization using low-frequency DAS signal. The Leading Edge, 2017, 36(12): 975-980.

[6] MAXWELL S C. Microseismic hydraulic fracture imaging: The path toward optimizing shale gas production. The Leading Edge, 2011, 30(3): 340-346.

[7] KORSMEYER R W, GURNY R, DOELKER E, et al. Mechanisms of solute release from porous hydrophilic polymers. International Journal of Pharmaceutics, 1983, 15(1): 25-35.

[8] RITGER P L, PEPPAS N A. A simple equation for description of solute release I. Fickian and non-Fickian release. Journal of Controlled Release, 1987, 5(1): 23-36.

[9] VAN GENUCHTEN M T, ALVES W J. Analytical solutions of the one-dimensional convective-dispersive solute transport equation. USDA Technical Bulletin No. 1661, 1982.

[10] SHOOK G M, POPE G A, ASAKAWA K. Determining reservoir properties and flood performance from tracer test analysis. SPE 124614, SPE ATCE, New Orleans, LA, 2009. https://doi.org/10.2118/124614-MS

[11] ZHAO B, PANTHI K, MOHANTY K K. Tracer eluting proppants for hydraulic fracture characterization. Journal of Petroleum Science and Engineering, 2020, 190: 107048.

[12] ZHOU Y, LIU H, GAO J, et al. Coated proppants with self-suspension and tracer slow-release functions. Journal of Petroleum Science and Engineering, 2022, 208: 109645.

[13] LI N, CHENG Q, GONG Z, et al. Release kinetics of rare earth tracer from polymer-coated proppants. Geoenergy Science and Engineering, 2023, 227: 211782.

[14] GONG Z, LI N, KANG W, et al. Novel oleophilic tracer-slow-released proppant for monitoring the oil production contribution. Fuel, 2024, 364: 130945.

[15] FONTALVO E M, OLIVEIRA M C, SCHOEGGL F, et al. Physical interpretation of interwell partitioning tracer tests. Transport in Porous Media, 2025, 152: 21-45.

[16] VELASCO-LOZANO M, BALHOFF M, DIAZ-PAULINO L, et al. Modeling of chemical tracers for two-phase flow in advective-dominated porous media. SPE Journal, 2024, 29(7): 3718-3731.

[17] MAZO A B, KHAMIDULLIN M R, POTASHEV K A, et al. Mathematical model of water- and oil-soluble tracers transfer in multistage hydraulic fracturing. Fluid Dynamics, 2024, 59(3): 427-443.

# Figure Captions

Fig. 1.  Schematic of breakthrough-curve (BTC) generation in the core displacement experiment. During the 96-h shut-in, tracer diffuses from the epoxy matrix of the ESP-T pack and accumulates in the pore space of the pack; on flowback at the pump setting, the accumulated slug is swept to the sampling point as a Gaussian pulse (Source I), followed by the sustained diffusion-controlled release tail (Source II).

Fig. 2.  ESP-T characterization summary (four panels) verifying the material prerequisites of the coupled model: SEM-EDS cross-section showing Fe distributed throughout the particle; TGA/DTG thermogram with decomposition onset 357 °C; K-P batch release fits at 30–120 °C (n = 0.45–0.85); water contact angle (104.6°) and oil/water filtration time ratio 5.53 demonstrating oil-phase selectivity.

Fig. 3.  K-P batch release kinetics (30–120 °C, Mt/M∞ < 0.6) and overlay of the single-process candidate models on the measured BTC: each single-process model reproduces only one feature of the curve — the peak or the tail — and none captures both.

Fig. 4.  Model selection. (a) Fits of the five candidate models — dual-component tanh-blended, single Gaussian, single erfc, exponential decay, and Korsmeyer–Peppas power law — to the 21-point single-phase BTC. (b) ΔAICc ranking: only the dual-component model is supported (ΔAICc = 0; every alternative ≥ 32.66).

Fig. 5.  BTC decomposition and model validation. (a) Fitted dual-component model (Eq. 1) with component decomposition: the shaded Gaussian pulse carries 53% of the integrated signal and the shaded erfc tail carries 47%, joined across the tanh transition centered on the fitted crossover t₀ = 25.66 min; residuals beneath the fit show no systematic structure. (b) Fitted flow rate Q = 0.46 ± 0.02 mL/min from four independent global searches versus the pump setting of 0.50 mL/min (8% deviation).

Fig. 6.  Time-of-arrival comparison. (a) Measured BTC with peak-time (t_peak = 15 min), half-peak (t_half ≈ 5 min), and first-moment markers. (b) Flow-rate estimates from the peak-time (1.31 mL/min, +162%), half-peak (3.93 mL/min, +685%), and first-moment (0.53 mL/min, +5.8%) methods and the coupled model (0.46 mL/min, −8%) relative to the pump setting (0.50 mL/min). (c) Qualitative comparison of method capabilities.

Fig. 7.  Independent corroboration of the two-process picture. Left panel: K-P fits to the static batch release data (n = 0.45–0.85, non-Fickian release). Right panel: fitted single-phase BTC annotated with the recovered Peclet number (Pe = x/α = 0.934, dispersion-dominated transport).

Fig. 8.  Transition-width sensitivity. (a) erfc tail fraction versus σ over a six-fold range (0.5×–3.0× of the fitted value 3.96 min); the fraction stays within 46.7–47.5%. (b) Component decomposition at the fitted σ. (c) Fitted BTCs at five σ values.

Fig. 9.  Two-phase production allocation. (a) Oil-phase tracer concentration C_oil versus total flow rate at OWR 4:1, 1:1, and 1:4 (dilution effect). (b) Oil-phase tracer mass flux F_O = C_oil × Q_oil versus total flow rate: flat within each OWR, confirming the mass balance. (c) Normalized flux F_O/F_O,ref versus oil flow rate Q_oil, collapsing the three OWR families onto a single trend (Pearson r = 0.97, RMSD = 8.3%).

Fig. 10.  Field deployment pathway: (1) stage tagging with distinct metal/rare-earth dopants; (2) single shut-in after fracturing; (3) flowback sampling at intervals guided by Eq. (1); (4) per-stage BTC fitting to recover stage flow rates Q_i; (5) production allocation via the flux method during steady production; (6) well-spacing and completion optimization from per-stage rate trends.

# Tables

[Table 1] Parameter set of the coupled release–transport model (Eq. 1) — Section 2.3.

[Table 2] Material prerequisites of the coupled release–transport model and the evidence supporting each — Section 3.2.

[Table 3] Model-selection statistics for the five candidate models fitted to the 21-point single-phase BTC — Section 4.1.